AADT Verification Categories

Note: Take this info with a couple grains of salt.  There has been some adjustment of AADT values since these categories were established.  Most notably, 100 has been added to the values of any (about 350) records that were re-evaluated upon determining that a pair of NodeLegs, corresponding to the opposite ends of their originating road centerline arc, had different AADT values after the initial review.

Many of those values have since been updated, as well as additional NodeLegs that were

- isCalpt = 1 AND AADT_MM_Break = 1 AND AADT_Temp_001 <> AADT_SJoin (9)

  - Manual (7 not SJoin)

- isCalpt = 1 AND AADT_MM_Break = 1 AND AADT_Temp_001 = AADT_SJoin (4551)

  - Autopopulated with AADT_SJoin or AADT_Temp_001

- isCalpt = 0 AND AADT_MM_Break = 1 AND AADT_Temp_001 <> AADT_Temp_05 (6)

  - Manual

- isCalpt = 0 AND AADT_MM_Break = 1 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin (610)

  - Autopopulated with AADT_SJoin or AADT_Temp_001

- isCalpt = 1 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_SJoin = AADT_Temp_001 AND CountNodesNear_265 IS NULL (97), so assume that the nearby split should have occurred at this node, not at another nearby node.  (97, 3 exceptions)

  - Autopopulated with AADT_Temp_05

- isCalpt = 1 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_SJoin = AADT_Temp_001 AND CountNodesNear_265 IS NOT NULL (304) Same as above, but where another node is close, so AADT_Temp_05 potentially reaches past neighboring node with a legitimate change in AADT. (If close to break in that direction, flag as 6, but keep close value, don’t use AADT_Temp_05 unless manually flagged as 21)

  - Autopopulated with AADT_Temp_001 (two exceptions)

- isCalpt = 1 AND AADT_MM_Break = 0 AND AADT_Temp_0011 <> AADT_Temp_0009 (97) Simple case of AADT measure being 0.001 mi different than the measure on a calibration point.

  - Autopopulated with AADT_Temp_0011 (AADT_Temp_001 is Null)

- isCalpt = 1 AND AADT_MM_Break = 0 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin (6020) Pretty much all AADT values match, so the break is either so close that it’s not found with 0.00 buffer,or is further than 0.05 mi away.

  - Autopopulated with AADT_SJoin

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin (9632) Pretty much all the AADT values match. Easy!

  - Autopopulated with AADT_Temp_001

- AADT_SJoin IS NOT NULL AND AADT_SJoin <> 0 AND AADT_Temp_001 IS NULL (7164) These records do not have TWN_MM

  - Autopopulated with AADT_SJoin

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 <> AADT_SJoin (35)

  - Use AADT_Temp_05 value

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 <> AADT_SJoin (35) Use AADT_SJoin value

  - Manual

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_Temp_001 <> AADT_SJoin (67)

  - Use AADT_SJoin value

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_Temp_001 <> AADT_SJoin (67)

  - Don’t use AADT_SJoin value (Manual)

- Manual observation of leg off AADT count range

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin AND CountNodesNear_265 >=1 (636) (Like 6, but isCalpt = 0 here) (two exceptions)

  - Autopopulate with AADT_Temp_0011

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin AND CountNodesNear_265 IS NULL (48) Same as above, but CountNearNodes_265 IS NULL, so the Most likely reason the furthest measure is different is if the split is not very close, because there aren’t any close nodes either.  Unless calibration was way off or there’s a node just beyond 265’ that has a legit change in AADT

  - Manually populated with AADT_Temp_05 (30)

- isCalpt = 0 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 AND AADT_Temp_001 = AADT_SJoin AND CountNodesNear_265 IS NULL (48) Same as 17, but manually reviewed to need AADT_SJoin

  - Manually populated with AADT_SJoin (18)

- Copied from AADT_Revised

- isCalpt = 0 AND AADT_MM_Break = 1 AND AADT_Temp_001 = AADT_Temp_05 AND AADT_Temp_001 <> AADT_SJoin (305) Like case 4 but the opposite leg (as long as other nodes aren’t a factor)

  - Autopopulated with AADT_Temp_001 or AADT_Temp_05

- isCalpt = 1 AND AADT_MM_Break = 0 AND AADT_Temp_001 <> AADT_Temp_05 Like case 6, but use AADT_Temp_05 even though there are nodes near, because the break isn’t supposed to be at those nodes.

- Manual correction of error not detected automatically

100-125. NodeLeg records from any category that originated from the same road centerline arc, but were found to have different AADT values after the above review.  Sometimes the difference is due to a legitimate difference in the AADT values, but where there is no split in the road centerline data to coincide with a break in the AADT records. This also indicates a significant flow of traffic along a leg that is not in the dataset.