MajorMinorTier

Designating Major_Legs and Minor_Legs

The field MajorMinorTier indicates categories of Nodes that have similar structure according to the NodeDesc field (for pattern exploration purposes initially) and eventually more specific categories according to how the Major and Minor approaches fit into the context of the order of sorted NodeLegs feature, as indicated by which approaches were essentially across from each other  (180 degrees apart).

The NodeLegs are sorted by NodeID (increasing), AADT (decreasing), FUNCL_Sortable (increasing, with the value 10 replacing zeros so they sort last as the lowest functional class), and TWN_LR (so that ramps and approaches sort after their associated route).

The Node dataset fields with values sensitive to the sorting of the NodeLegs dataset are:

NodeLegID_1, … NodeLegID_6 (6 fields total)

TWN_LR_1, … TWN_LR_6 (6 fields total)

NodeDesc

NodeDescFUNCL

FUNCL_MonoFlag

TWNLR_Count

NodeDescAADT

AADT_MonoFlag

NodeDescCalpts

NodeDescCompassAngle

NodeDescTWNLR

NodeDescLegID

Angle_1_2, Angle_1_3, etc… (6 fields, combos for the first 4 legs)

Caution: Most importantly, the fields Major_Leg, Minor_Leg, Major_Leg_2, Minor_Leg_2, and Minor_Leg_3 depend on the order of the NodeLegID_n fields, but are populated manually instead of via a Python Script.  Therefore, if the records of the NodeLeg dataset are re-sorted such that the NodeLegID’s change order within a Node, then the values for Major and Minor legs may change, and generally from correct to incorrect.

So far, only at-grade Nodes with at least three legs, where at least one leg has a non-zero functional class (i.e. is a route in the route feature class ‘lrs_route_twn’), and where all legs have a non-zero AADT value have been considered. (Some cases with a single local route (TWN_LR LIKE ‘L%’) with one or two legs have also been considered)

The NodeDesc field provides information regarding route (as defined by TWN_LR) continuity (capital letters without spaces) and its relationship with the AADT of each leg (highest AADT to lowest).  Ideally the legs with the two highest AADT values belong to the same TWN_LR, though there are many cases where a cross street has the second highest volume.

The Angle_1_2, etc… fields provide some information about whether the two legs continue through the intersection (angles closer to 180) or represent turning movements (angles closer to 90)

Together, the NodeDesc and Angle_i_j fields provide a generalized description of the approaches at each Node, which is often useful for determining which legs are Major and Minor at each Node. Although the Major route is always defined by the leg with the highest AADT, the Minor route is defined as intersecting the major route, and can therefore be either the second highest AADT or the third highest, depending on the geometry of through movement at each Node (defined by STOP control at partially controlled intersections).

AADT approach issues:

If there is a T intersection where the base of the “T” has the highest AADT, it might be a simple exception, indicate a through-turn-maneuver, an error in assigning AADT to one or more legs, or be an artifact of how AADT is recorded when there are approaches.  Check for nearby approaches that are not part of the “T”, but whose traffic was included with the AADT on the “base” leg.  This will also affect how Major and Minor are assigned to the nearby intersections of the approach with the mainline. The intersection between V022A and V004A illustrates these issues:

At this time, Major and Minor are defined in terms of their usefulness in Safety Analyst; more specifically, how well they match the scenarios underlying each available Safety Performance Function (SPF).  Many scenarios in the data violate the SPF assumptions, and the challenge in such cases is to determine which SPF is the most appropriate substitute, and to define Major and Minor accordingly. Nodes that have been recognized as exceptions within their tiers below have comments in the field “QAQC_Note”. Not all nodes in every tier have been reviewed

None of the following scenarios for defining the Major and Minor routes take into account unmapped legs (e.g. shopping center entrances, other unmapped signalized approaches, etc.) or the possibility that a node represents only a portion of a more complex intersection.  The field “Complex” is a conservative indicator of possible complex intersections (with a high likelihood of false positives), and therefore these nodes have been included within the tier grouping, but not necessarily have had all of their Major/Minor Legs determined.

Nodes that have been assigned to a MajorMinorTier have not necessarily been reviewed. Null values in the Minor_Leg field indicates intersections where some big question remains. These are mostly Tier 4 and 7 Nodes. Some potentially Complex or missing-leg Nodes have several Major and Minor legs identified if the context suggests it won’t change when addressed further, but Minor_Leg_2 has been left Null, indicating that it should have a value.  Minor_Leg_3 values have been left Null unless Complex = 0, but have not yet been reviewed for possible un-digitized legs.

MajorMinorTier Dictionary

- Manual, random

- NodeLegCount = 4 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' '

  - (79, but two fail angle conditions that were added later, have smallest NodeMinAngle)

  - Two adjacent letters in NodeDesc indicate that Leg1 and Leg2 belong to the same TWN_LR code. Angle conditions ensure that Leg2 (likely Major2) does not reflect a turning movement.

  - Major-Major-Minor-Minor (shorthand for the following 4 designations)

  - Major_Leg = 1

  - Major_Leg_2 = 2

  - Minor_Leg = 3

  - Minor_Leg_2 = 4

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND Angle_1_2  > 170 AND NodeMinAngle > 40

  - (381)

  - Major-Major-Minor

  - Leg1 and Leg2 have the most direct path through the intersection, and have same TWN_LR, so are very likely Major1 and Major 2. Minor1 is Leg3. If Complex = <> 0, it needs review

  - Major_Leg = 1

  - Major_Leg_2 = 2

  - Minor_Leg = 3

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND Angle_1_2 < Angle_2_3

  - (11)

  - Major - - (depends…)

  - Needs review because angles suggest that leg with highest AADT is a route with a turning maneuver or a sharp curve, whereas Leg3 continues or has less turning and is a potential alternative candidate for Major 2.

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND MajorMinorTier NOT IN (2, 3, 13, 14)

  - (16) Sub-scenario remaining after angle sub-scenarios 2 and 3, 13, and 14 are covered. Needs review...

- (5 & 50) NodeLegCount IN (3,4) AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) = ' ' AND NodeDesc NOT LIKE '%+1%' AND SUBSTRING(NodeDesc FROM 6 FOR 1) <> ' '

  - (45) Mostly a potpourri of Y and T scenarios. Similar to scenarios 1-4, but where Leg2 does not have the same TWN_LR code as Leg1. NodeDesc has form 'A BB …', where A and B are different TWN_LR codes.  In some cases, one leg has artificially high AADT because the value represents the total of that leg plus another, that may or may not be within the zone of influence. Sometimes there is legitimately a high degree of turning movement, and Stop control may accommodate this, resulting in non-standard intersections. See QAQC notes. Values marked MajorMinorTier = 50 have had Major and Minor legs assigned manually after a review, but as NodeLegs, not IntersectionLegs.

- NodeLegCount IN (3,4) AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) = ' ' AND SUBSTRING(NodeDesc FROM 6 FOR 1) = ' ' AND NodeDesc NOT LIKE '%+1%'

  - (15) Easy, first three TWN_LR codes are different, so this scenario sticks to the Major-Major-Minor- AADT sorting rules as long as Angle_1_2 is close to 180 degrees, or at least significantly larger than both the other angles. (So far, all the three-leg intersections in this group are like this, and 2/4 of the four-leg intersections are).

- NodeLegCount IN (3,4) AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) = ' ' AND SUBSTRING(NodeDesc FROM 6 FOR 1) = ' ' AND NodeDesc LIKE '%+1%'

  - Either the second leg of the first TWN_LR has a lower volume and continues straight through the intersection (less complicated), or the

  - (9) Four-leg intersections – if Angle_1_2 and Angle_3_4 are both about 180, legs are Major-Major-Minor-Minor (1, 2, 3, 4) and the first TWN_LR has a turn maneuver (the +1 indicates the second leg of the first TWN_LR was not the second highest AADT, and not at 180 deg indicates it wasn’t straight through).

  - If Angle_1_3 and Angle_2_4 are both about 180 deg, have Major-Minor-Major-Minor (1, 3, 2, 4)

  - (84)Three-leg intersections – If Angle_1_2 is close to 180 and Angle_1_3 is close to 90 deg, have Major-Major-Minor sorting, and the first TWN_LR has a turn maneuver with a decrease in traffic. Since both AADT and 180 deg angle of first two legs suggest functional “thoroughness” different from TWN_LR codes, maybe less likely STOP control favors a different interpretation. On the other hand, if another angle is about 180 deg, then things are weirder. Either base of T has highest count, or TWN_LR turns one way while traffic turns the other, all of which make defining Major/Minor more ambiguous if considering factors other than AADT and through-geometry.

  - If Angle_1_2 is close to 90 deg, then there is more turning movement than through movement, so extra potential for muddy definitions of Major and Minor.

- NodeLegCount > 2 AND AADT_NullCount = 0 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND MajorMinorTier IS NULL

  - (10) The remaining potentiall intersection nodes with AADT on all legs

  - Major-Minor-Minor-Major (1, 4, 2, 3) because Angle_1_4 about 180

- NodeLegCount = 4 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 1 AND SubCat <> 'mix' AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' '

  - (105) Need complex review. This scenario is straight forward as long as either Angle_1_2 or Angle_1_3 is close to 180, then there is a clear Major route, and the Minor approaches are the remaining two legs (1, 2, 3, 4) or (1, 3, 2, 4) as long as the angle between the Minor approaches is greater than the angle between Major and Minor approaches.

- NodeLegCount = 4 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 2 AND SubCat NOT IN ('mix', 'mixP', 'aotclass', 'aotclassP') AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND NodeDesc NOT LIKE '%F%' AND NodeDesc NOT LIKE '%a%' AND TWNLR_Count = 2

  - (158) Crossroads where at least one route has FUNCL NOT IN (1-6), allowing for the minor route to be local or otherwise (without AADT value). Fed Aid route and local through road. Only one case where Angle_3_4 isn't 180, but seems ok.

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 1 AND SubCat <> 'mix' AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND Angle_1_2 > 170 AND NodeMinAngle > 60 AND Complex = 0

  - (6822 with complex and all angles, 5428 restricting angles, and 3772 eliminating potentially complex intersections too) Single FedAid route with intersected by a local road...assuming the intersection is simple. Unless a complex node in this category is a principal node, it's minor route is unlikely to be the intersection's minor route,  Note: There has been essentially no complex review Clear Major/Minor assigning with angle restrictions. What about the other cases?

  - One angle close to 180 degrees, the other two

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 1 AND SubCat <> 'mix' AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND ABS (90 - Angle_1_2) < 15 AND Complex = 0

  - (27) (41 restricting angles, and 27 eliminating potentially complex intersections too) Not reviewed

  - Distinct subset that would be part of scenario 11 without angle restrictions. The FedAid route likely has a turning maneuver.  Chance of AADT errors if node not a Calpt

  - 107/182 with these two angle secnarios AND (Angle_1_3 > 170 OR Angle_2_3 > 170)

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND Angle_1_2  > 170 AND NodeMinAngle <= 40

  - (32)  Major-Major-Minor

  - including complex and potentially complex) Same as scenario 2 above, but with more extreme Minor approach angle (< 40 deg). Major and Minor approaches likely identified reliably for simple intersections as long as Safety Analyst doesn’t need to be accommodated. Complex intersections need review.

  - The constraint on Angle_1_2 helps ensure that both legs should be categorized as Major, even though the minor approach can potentially more directly line up across from one of the Major approaches. That small leeway in straightness of the Major route and skewness of the Minor route can make this seem like a mild version of scenario 3.

- NodeLegCount = 3 AND NotAtGrade = 0 AND SecFlag = 0 AND AADT_NullCount = 0 AND SUBSTRING(NodeDesc FROM 2 FOR 1) <> ' ' AND Angle_1_2  < 171 AND Angle_1_2 > Angle_1_3 AND Angle_1_2 > Angle_2_3

  - (32) Major-Major-Minor (1, 2, 3)

  - Same as scenarios 2 and 13 above, without restricting the “Major” route to close to 180 degree angle, but still making sure that angle between the Major approaches is greater than the angle between the Minor approach and either of the Major approaches (which might make it more appropriate to switch one Major approach to Minor, depending on best handling for Safety Analyst.

- NodeLegCount = 4 AND NotAtGrade = 0 AND SecFlag = 0 AND TWNLR_Count = 2 AND NodeDesc NOT LIKE 'L%' AND NodeDesc NOT LIKE '%a%' AND NodeDesc NOT LIKE '%F%' AND SubCat NOT IN ('mix', 'mixP', 'aotclass', 'aotclassP') AND (Angle_1_2 > 170) AND AADT_NullCount = 4 AND NodeDescFUNCL <> ‘6 6 6 6 ’

  - (41) Major-Major-Minor-Minor (1, 2, 3, 4)

  - Crossroads where FUNCL differentiates between Major and Minor, in absence of AADT data for any approaches

  - Similar to scenarios 1, 10, except using FUNCL as proxy for AADT

  - 3 intersections have same FUNCL for all 4 legs, and were omitted from this scenario

- NodeLegCount =3 AND NotAtGrade = 0 AND SecFlag = 0 AND TWNLR_Count = 2 AND FedAidFlag = 1 AND NodeDesc NOT LIKE '%a%' AND NodeDesc NOT LIKE '%F%' AND NodeDesc NOT LIKE '%r%' AND SubCat NOT IN ('mix', 'mixP', 'aotclass', 'aotclassP') AND (Angle_1_2 > 170) AND NodeMinAngle > 60 AND MajorMinorTier IS NULL

  - (802) Major-Major-Minor (1, 2, 3)

  - Three-leg intersections that are more T-like than Y-like (because of ~180 angle), but filtering out approaches, ramps, and potentially complex intersections

- NodeLegCount =3 AND NotAtGrade = 0 AND SecFlag = 0 AND TWNLR_Count = 2 AND FedAidFlag = 1 AND NodeDesc NOT LIKE '%a%' AND NodeDesc NOT LIKE '%F%' AND NodeDesc NOT LIKE '%r%' AND (Angle_1_2 > 170) AND NodeMinAngle > 60 AND (NodeDesc LIKE '% 8'  OR NodeDesc LIKE '% 9' OR NodeDesc LIKE '% 60' OR NodeDesc LIKE '% 97' OR NodeDesc LIKE '% 5' OR NodeDesc LIKE '% 6')

  - (3130) Major-Major-Minor (1, 2, 3)

  - Same as scenario 16, except conditionally allow for minor ‘routes’ that don’t have route codes, as long as they belong to limited set of AOTCLASS values (Private, Military, National Forest, State Forest, Discontinued Now Private). Didn’t include class 4 town highways because of high likelihood that they could be untraveled, and SurfactType is not currently joined to NodeLegs